MJM MOLTEN-SALT SMR

Neutronics Solver v1 + PINN Surrogate

COMPREHENSIVE ASSESSMENT REPORT

Strengths, Weaknesses, and Physics Fidelity Evaluation

D4C Industries  |  ChE 612 Dragons of the West  |  April 2026

# OVERALL SCORECARD

The following grades evaluate each subsystem of mjm_solver_v1.py and mjm_pinn_train.py against three criteria: (1) physical correctness relative to ORNL-TM-730 Fuel C benchmark targets, (2) numerical implementation quality, and (3) fitness for use as a PINN training data source.

| Category | Score | Notes |
| --- | --- | --- |
| Solver numerical structure (FD, power iteration) | A | Converges in 11 iterations to ¦dk/k¦ < 5e-7; sparse matrix correct |
| FLiBe thermophysical property model | A- | Correctly sourced from Benes & Konings (2009); all 5 properties implemented |
| Flux field spatial shape | B+ | J0 radial, cosine axial; axial MAE 0.028; physically sensible |
| Critical dimension search (k_eff = 1.03) | B | Brent solver works; R=23.6cm, H=47.2cm consistent with 1-group estimates |
| Delayed neutron data source | B | ORNL-TM-730 Table 6.1 Keepin data correctly transcribed |
| Flux magnitude / power normalisation | D | 3.58e14 vs 3.29e13 benchmark (10.9x error) — fission rate integral wrong |
| Temperature coefficient magnitude | D | -53 pcm/K computed vs -12.53 pcm/K ORNL-TM-730 (4.3x too large) |
| Delayed neutron reduction (beta_eff/beta_static) | D | 0.982 computed vs 0.544 ORNL benchmark (formula logic error) |
| Group-2 diffusion coefficient D2 | D | 1.54 cm computed; physical range 0.30-0.50 cm (3x too large) |
| k_inf plausibility | C- | 1.998 computed; expected 1.3-1.5 for 4% UF4 in FLiBe at 19.75% HALEU |
| Doppler broadening model | C | Applied to thermal group only; fast resonance absorption (U-238) missing |
| Removal cross section Sigma_r1 | C | All group-1 scatter treated as downscatter; within-group scatter ignored |
| Vacuum boundary conditions | C | Zero-flux at physical boundary; extrapolation length (0.97 cm fast) missing |
| PINN flux shape surrogate | B | MAE 1.3-1.5% on normalised fields; spatial topology well-learned |
| PINN k_eff surrogate | F | 25,400 pcm bias at design point; training distribution heavily supercritical |
| PINN physics enforcement (PDE residual) | F | No PDE loss; pure data interpolator; no extrapolation guarantee |
| Training dataset quality | C+ | 100 samples, 11% near-critical, no flux field arrays stored; LHS method correct |
| Solver speed / scalability | A | 30ms at nr=20/nz=30; 228ms at nr=40/nz=60; suitable for 1000-sample sweeps |

# SECTION 1 — WHAT THIS DOES WELL

## 1.1  Numerical Solver Core — Power Iteration

The finite-difference power iteration engine is well-implemented. The key metrics:

- Convergence: 11 iterations from a cosine initial guess to |dk/k| < 5e-7 at the design point. This is tight — most reactor codes use 1e-6 as the convergence tolerance, so the solver is achieving sub-tolerance comfortably.

- Sparse matrix assembly: _build_diffusion_operator() correctly uses scipy.sparse CSR format. The r-weighted finite difference stencil for the cylindrical radial term (D*r_face / (r_center * dr^2)) is correctly implemented with harmonic-mean face diffusion coefficients.

- Volume elements: the power normalization integral correctly uses 2*pi*r_i*dr*dz as the cell volume in cylindrical coordinates. This is a common error in cylindrical FD codes — the solver gets it right.

- Eigenvalue update: k_new = k * (sum of new fission source) / (sum of old fission source) is the standard power iteration update. It is correctly applied.

- Flux floor: phi = max(phi, 1e-10) prevents negative flux during early iterations.

## 1.2  FLiBe Thermophysical Property Model

All five FLiBe properties are sourced from the correct primary reference (Benes & Konings, J. Fluorine Chemistry 130, 2009) with proper equation attribution. Computed values at 900K:

| Property | Computed (900K) | Literature Range | Assessment |
| --- | --- | --- | --- |
| Density | 1.707 g/cm^3 | 1.70-1.72 g/cm^3 | Correct — Cantor (1971) correlation |
| Viscosity | 7.52 mPa*s | 6-10 mPa*s at 900K | Correct — Cohen & Jones (1957) |
| Heat capacity | 2.39 J/g*K | ~2.39 J/g*K | Correct — Douglas & Payne (1969) |
| Thermal conductivity | 1.1 W/m*K | 1.0-1.2 W/m*K | Correct midpoint estimate |
| Vapor pressure | ~1 Pa at 900K | Very low at MSR temp | Correct order of magnitude |

- Melting margin of 141 K above T_inlet (873 K - 732.3 K) is correctly flagged and checked against operating conditions.

- Temperature dependence of density is included in SaltMaterial via FLiBeProperties.density_gcc(T), so salt mass is consistent with operating temperature.

## 1.3  Flux Field Spatial Shape

Despite the absolute magnitude error from the normalization bug, the normalised flux shape is physically reasonable and internally consistent:

- Radial profile: peaks at r=0, drops to 0.66 at r=R/2 and to 0.04 at r=R. This is consistent with the J0(2.405*r/R) Bessel function shape for a critical cylinder, which would give J0(2.405/2) = 0.61 and J0(2.405) = 0. The small nonzero edge value (0.04) is an artefact of the zero-flux BC being applied at the physical rather than extrapolated boundary.

- Axial profile: cosine MAE of 0.028 against an ideal cos(pi*z/H) reference. This is acceptably small — the two-group flux deviates slightly from pure cosine because the fast-to-thermal downscattering couples the groups axially.

- Peak location: fast and thermal flux both peak near (r~0.6 cm, z~23-24 cm), consistent with the geometric centroid of the cylinder.

- Peaking factor of 3.35-3.44 is physically plausible for a bare cylinder with no reflector.

## 1.4  Delayed Neutron Data Transcription

The six-group delayed neutron data for U-235 at lines 146-154 match Keepin, Wimett & Zeigler data as tabulated in ORNL-TM-730 Table 6.1. Half-lives and fractional yields are correctly transcribed:

| Group | Half-life (s) — Code | Half-life (s) — ORNL | beta_i — Code | beta_i — ORNL |
| --- | --- | --- | --- | --- |
| 1 | 55.7 | 55.7 | 2.11e-4 | 2.11e-4 |
| 2 | 22.7 | 22.7 | 1.402e-3 | 1.402e-3 |
| 3 | 6.22 | 6.22 | 1.254e-3 | 1.254e-3 |
| 4 | 2.30 | 2.30 | 2.528e-3 | 2.528e-3 |
| 5 | 0.61 | 0.61 | 7.40e-4 | 7.40e-4 |
| 6 | 0.23 | 0.23 | 2.70e-4 | 2.70e-4 |

beta_static = 0.00641 (640 pcm) vs ORNL 0.00666 (666 pcm) — 2.4% low. This is a known bias from importance-weighting effects on the static beta; acceptable for a 2-group code.

## 1.5  Material Model Structure

SaltMaterial.compute_number_densities() correctly handles the mixed salt composition. Number density calculation for U-235:

N(U235) = rho_salt * N_A / M_mix * f_salt * enrichment * x_UF4

At 900K, 4% UF4, 19.75% enrichment: N(U235) = 7.998e19 atoms/cm^3. This is internally consistent with the 1.707 g/cm^3 salt density at temperature. The UF4 density correction (1 + 1.5*x) is a reasonable first-order approximation for the heavier solute.

- Avogadro constant: 0.6022e24 — correct.

- Mass of F-19: 18.998 amu — correct.

- Mass of Li-7: 7.016 amu — correct (uses Li-7, not natural Li).

- Isotopic attribution: Li-7 only (not Li-6) which is correct for FLiBe which uses enriched Li-7 to minimise tritium production.

## 1.6  Solver Speed

The solver achieves 30ms per solve at nr=20, nz=30 (600 cells) and 228ms at nr=40, nz=60 (2400 cells) on CPU-only hardware. This is fast enough to generate 1000-sample training datasets in under 4 minutes, and 100 samples in 12 seconds as demonstrated. The sparse matrix formulation (scipy.sparse CSR + spsolve) is the correct approach for this problem size and avoids dense matrix inversion.

## 1.7  PINN Architecture and Flux Shape Learning

The PINN architecture (6-layer fully connected, 128 neurons, tanh activations, Xavier init) is appropriate for a smooth physics function. The ReLU output activation on phi1/phi2 correctly enforces non-negativity. Training results:

- phi1 normalised MAE: 0.0128 — the network correctly learns the Bessel-cosine spatial shape

- phi2 normalised MAE: 0.0150 — thermal group also well-learned

- Midplane peak: phi1_PINN[0,mz] = 0.9988, phi2_PINN[0,mz] = 0.9979 vs FD = 1.0000 (0.1% error at centerline)

- Training and validation MSE track closely (no overfit): train=4.9e-5, val=4.9e-5 at convergence

- Cosine annealing LR schedule correctly reduces step size over 3000 epochs

## 1.8  Critical Radius Search

find_critical_radius() using scipy.optimize.brentq is the correct tool for this 1D root-finding problem. The solver brackets [k_lo, k_hi] before calling Brentq, and re-evaluates at the root with a finer mesh. This gives a defensible R_crit = 23.6 cm, H_crit = 47.2 cm at k_eff = 1.030 with 3% excess reactivity as designed.

# SECTION 2 — WHAT NEEDS IMPROVEMENT

## 2.1  Flux Magnitude Wrong by ~11x — Normalization Bug

This is the highest-priority defect. The power normalization integral on lines 477-486 collapses the 2-group fission rate to a 1-group scalar by summing cross sections and fluxes before multiplying:

Sf = Sigma_f1 + Sigma_f2

fission_rate = Sf * (phi1 + phi2) * dV

The correct group-wise integral is Sigma_f1*phi1 + Sigma_f2*phi2. The cross-group terms Sigma_f1*phi2 and Sigma_f2*phi1 inflate the denominator by approximately 2x, halving the norm_factor and producing 11x too high a flux. Every downstream physical quantity — peak flux, peak power density, peaking factor interpretation — carries this error.

| Quantity | Computed | ORNL-TM-730 Target | Error |
| --- | --- | --- | --- |
| Peak thermal flux at 10 MW | 3.58e14 n/cm^2/s | 3.29e13 n/cm^2/s | 10.9x |
| Peak power density | 405 W/cm^3 | ~37 W/cm^3 (est.) | ~11x |
| Avg power density (P/V) | 121 W/cm^3 | 121 W/cm^3 | Correct — unaffected |

## 2.2  Temperature Coefficient 4.3x Too Large

Computed alpha_T = -53.4 pcm/K vs ORNL-TM-730 Fuel C value of -12.53 pcm/K. The code computes alpha_T via central difference: (k_hi - k_lo) / (2*dT) / k_avg * 1e5, where dT=25K.

- k at 875K: 1.03871, k at 925K: 1.01134. Delta-k over 50K = -0.02737. This gives dk/dT = -5.47e-4 /K and alpha = -53.4 pcm/K.

Three compounding causes:

- Doppler applied to group-2 only (lines 306-309). U-238 resonance absorption in the fast/epithermal region is the primary Doppler mechanism but is excluded. This makes k more temperature-sensitive than it should be because only the thermal group XS change but they change too aggressively.

- Single global temperature. No spatial averaging means the full dT=50K perturbation is applied to all material uniformly, including the flux-weighted centroid where sensitivity is highest. A volume-averaged or flux-weighted temperature would give a smaller effective delta.

- D2 = 1.54 cm is 3x too large (see 2.4). An inflated D2 makes the thermal group leakage term D2*B^2*phi2 larger than it should be, amplifying the sensitivity of k to changes in thermal group absorption.

## 2.3  Delayed Neutron Reduction Factor 1.8x Wrong

Computed beta_eff/beta_static = 0.982 vs ORNL-TM-730 Fuel C value of 0.544. A ratio of 0.982 means the code computes almost no loss of delayed neutrons to the external loop — physically this would require essentially all precursors to decay inside the core before exiting, which requires t_core >> all precursor half-lives.

The thermal-hydraulic state: core residence time = 1.69 s, loop transit = 3.37 s. For group 2 (22.7s half-life), lambda*t_core = 0.693/22.7 * 1.69 = 0.052. The fraction in-core should be about (1-exp(-0.052))/(1-exp(-0.052+0.030)) ~ 0.37 — a 63% loss of that group's contribution. The code instead returns near-unity for all groups.

The bug: the frac_in_core formula is computed and then unconditionally overwritten by a second formula at lines 212-214. The second formula evaluates to near 1.0 for all groups with the given residence times. See Defect 8 in the companion report for the complete fix using the ORNL-TM-730 Section 6.1 formula.

## 2.4  Diffusion Coefficients D1 and D2 Are Wrong

Computed: D1 = 2.04 cm, D2 = 1.54 cm. Physical range for a FLiBe-moderated MSR:

| Parameter | Computed | Expected Range | Error Factor |
| --- | --- | --- | --- |
| D1 (fast group) | 2.04 cm | 1.2 - 2.0 cm | Marginally high |
| D2 (thermal group) | 1.54 cm | 0.30 - 0.50 cm | 3 - 5x too large |

D2 being 3-5x too large is a direct consequence of the Sigma_s12 = Sigma_s1 bug (Defect 5). The transport cross section for group 2 is Str2 = sum(n_i * sigma_s2_i * (1 - 2/(3*A_i))). Be-9 has sigma_s2 = 6 barns (reasonable), but the thermal scatter XS values in the database may also be systematically low, which would reduce Str2 and inflate D2 = 1/(3*Str2). The consequence is that the thermal group flux is more diffuse than physical reality — it penetrates further from the center and produces a broader peaking profile than the benchmark suggests.

## 2.5  k_inf = 1.998 Is Physically Implausible

k_inf should represent the multiplication factor of an infinite homogeneous medium with the same composition. For 4 mol% UF4 in FLiBe at 19.75% enrichment, published estimates for similar MSR compositions suggest k_inf in the range 1.3-1.6. The computed value of 1.998 implies that leakage is almost entirely responsible for keeping the 23.6 cm core from being far supercritical, which contradicts the intended dilute-fuel design philosophy of the MSRE.

Root causes:

- The k_inf formula at line 316 is: (nu*Sf1 + nu*Sf2 * Ss12/Sa2) / Sr1. This is a simplified 2-group k_inf that assumes all group-1 neutrons slow down to group 2, which overcounts the thermal fission contribution when Sigma_s12/Sa2 >> 1 (which it is here: 0.172/0.032 = 5.4).

- The correct 2-group k_inf is: (nu*Sf1 + nu*Sf2 * Sigma_s12/(Sigma_a2)) / Sigma_r1, but this is only valid if Sigma_r1 correctly represents absorption + downscatter only (not all scatter). Since Sigma_r1 = Sa1 + Ss12 = Sa1 + Ss1 (all scatter), the formula is simultaneously under-removing from group 1 and over-attributing fissions from group 2.

- Cross-section values may also be incorrect in absolute magnitude. The thermal fission XS for U-235 (sigma_f2 = 585 barns) is correct at 2200 m/s. However, using the 1/v Doppler scaling sqrt(T0/T) = sqrt(293/900) = 0.571 applied to a 2200-m/s value for a 900K spectrum is an approximation that can over- or under-estimate depending on the spectrum hardness.

## 2.6  PINN k_eff Surrogate Is Unusable at Design Point

k_eff prediction error of 25,400 pcm at the nominal design point renders the surrogate non-functional for any reactivity analysis. The cause is the training data distribution: mean k_eff = 1.348, std = 0.361, with only 11% of samples in the near-critical band 0.9-1.1. The PINN regresses toward the training mean rather than the physical design point value.

In operator learning terms: the network has learned a function valid over the full parameter hull but cannot resolve the narrow near-critical band with sufficient accuracy because its training gradient signal is dominated by the large-k tail of the distribution.

## 2.7  No Physics Constraints in PINN (PDE Residual Missing)

The PINN is trained purely on data. There is no automatic differentiation through the network to enforce the 2-group diffusion equations as a loss term. This means:

- The network can predict flux shapes that violate neutron balance at arbitrary (r,z) locations not seen in training.

- The network has no mechanism to extrapolate to geometries outside the training convex hull — it interpolates, and the quality of that interpolation degrades monotonically with distance from training samples.

- The k_eff prediction is completely decoupled from the flux field in the current implementation — the same output head predicts k alongside phi1 and phi2, but there is no eigenvalue consistency constraint enforcing that the predicted flux satisfies A*phi = k*F*phi.

## 2.8  Boundary Conditions Use Zero-Flux at Physical Surface

The FD operator implements zero-flux at r=R and z=0,H rather than at the extrapolated boundary r=R+delta, z=delta, z=H-delta. The vacuum extrapolation length is delta = 2*D/3:

| Group | D (cm) | delta = 2D/3 (cm) | Effect on k_eff |
| --- | --- | --- | --- |
| Group 1 (fast) | 2.04 | 1.36 cm | Significant — 5.8% of radius |
| Group 2 (thermal) | 1.54 | 1.03 cm | 4.4% of radius |

For the design point R=23.6 cm, an extrapolation length of 1.2 cm (average) represents a 5% underestimate of the effective radius. Since geometric buckling B_g^2 = (2.405/R_eff)^2 + (pi/H_eff)^2, this artificially inflates B_g^2, which suppresses k_eff by O(100-200 pcm) — not catastrophic but measurable and fixable trivially.

## 2.9  Training Dataset Is Missing Flux Field Arrays

pinn_training_data.json stores only {k_eff, k_inf, peaking_factor} as outputs — no phi1(r,z), phi2(r,z) arrays, no r_centers, no z_centers. This forces mjm_pinn_train.py to re-solve all 100 designs at training time (12 seconds), and means the training data cannot be used for offline analysis without the solver present. For a 500-sample target dataset with phi1/phi2 at nr=20, nz=30, the storage cost is 500 * 2 * 600 * 8 bytes = 4.8 MB — entirely manageable as JSON or npz.

## 2.10  Thermal-Hydraulics Model Has Unchecked Assumptions

compute_thermal_hydraulics() uses a fixed delta_T = 50K and a fixed hydraulic diameter D_h = 0.02 m (2 cm). These values are hardcoded estimates, not derived from the core geometry. The 2cm hydraulic diameter is for a channel-type MSR; for a homogeneous pool-type reactor like the MSRE/MJM concept, the correct D_h is the effective channel dimension of the graphite moderator matrix, which is not defined anywhere in the code. The flow velocity = 0.78 m/s and Re = 12,700 (turbulent) are sensitive to this assumption. No Nusselt-number-based heat transfer coefficient is computed, so the model cannot verify that adequate heat removal is achievable at the stated conditions.

# SECTION 3 — BENCHMARK COMPARISON AGAINST ORNL-TM-730 FUEL C

All targets are from ORNL-TM-730, Haubenreich et al. (1964). MJM is a different design (FLiBe/HALEU, 10 MW) vs MSRE Fuel C (FLiBe/U-235, 8 MW), so deviations of 20-30% are physically expected. Deviations of 2-10x indicate modeling errors.

| Parameter | MJM Computed | ORNL-TM-730 Fuel C | Deviation | Root Cause |
| --- | --- | --- | --- | --- |
| Peak thermal flux at 10MW | 3.58e14 n/cm^2/s | 3.29e13 n/cm^2/s | 10.9x HIGH | Normalization bug — Defect 1 |
| Temperature coefficient alpha_T | -53.4 pcm/K | -12.53 pcm/K | 4.3x LARGE | Doppler group-1 missing; flat T field |
| beta_eff/beta_static ratio | 0.982 | 0.544 | 1.8x HIGH | frac_in_core formula overwrite — Defect 8 |
| beta_static | 640 pcm | 666 pcm | 4% low | No importance weighting; acceptable |
| beta_eff (circulating) | 629 pcm | 362 pcm | 1.7x HIGH | Same as ratio error above |
| k_eff at design point | 1.020 | ~1.03 target | 1% low | Acceptable; within BC error |
| Prompt neutron lifetime Lambda | Not computed | 2.40e-4 s | Missing | Not implemented |
| Control rod total worth | Not computed | 5.7% delta-k/k | Missing | Not implemented |
| Peak power density | 405 W/cm^3 | ~37 W/cm^3 (est.) | ~11x HIGH | Normalization bug — Defect 2 |
| D2 (thermal diffusion coeff) | 1.54 cm | 0.30-0.50 cm | 3-5x HIGH | Sigma_s12 overcounting — Defect 5 |
| k_inf | 1.998 | ~1.3-1.5 (expected) | ~40% HIGH | XS model / k_inf formula error |

# SECTION 4 — RECOMMENDED IMPROVEMENT SEQUENCE

Ordered by impact/effort ratio. Items 1-4 fix the benchmark-failing outputs. Items 5-8 bring the physics to publication quality. Items 9-11 make the PINN a real surrogate.

| # | Action | Expected Benchmark Impact | Effort |
| --- | --- | --- | --- |
| 1 | Fix fission rate integral (Defect 1, lines 477-486) | Flux drops 10x to match ORNL-TM-730 within 20% | 5 min |
| 2 | Fix frac_in_core formula (Defect 8, lines 201-215) | beta_eff/beta_static converges from 0.982 to ~0.544 | 15 min |
| 3 | Add Doppler to group-1 XS for U235/U238 (Defect 6) | alpha_T magnitude reduces from 53 to ~20 pcm/K | 20 min |
| 4 | Fix Sigma_r1 / Ss12 with downscatter fraction (Defect 5) | D2 reduces, k_inf drops, XS model more physical | 30 min |
| 5 | Add extrapolation length to vacuum BC (Defect 9) | k_eff increases ~100-200 pcm; geometry more accurate | 30 min |
| 6 | Fix peak power density field (Defect 2, line 497) | Correct spatially resolved W/cm^3 field | 5 min |
| 7 | Store phi1/phi2 in training JSON (Defect 11) | Training data self-contained; eliminates re-solve cost | 30 min |
| 8 | Increase training samples to 500, stratify near-critical (Defect 4a, 12) | Training coverage improves; k_eff surrogate distribution fixed | 1 hour |
| 9 | Add separate KeffNet scalar head (Defect 4b) | k_eff error from 25,400 pcm to O(100 pcm) | 2 hours |
| 10 | Add PDE residual loss (Defect 3) | PINN becomes physics-constrained; enables extrapolation | 4 hours |
| 11 | Implement spatial T(r,z) outer iteration (Defect 7) | Self-consistent flux/temperature; correct alpha_T field | 8 hours |
| 12 | Implement prompt neutron lifetime Lambda | Adds missing kinetics parameter for control analysis | 2 hours |

# SECTION 5 — WHAT IS READY TO USE NOW

Despite the quantified defects, several components are production-ready without modification:

- FLiBe property model (FLiBeProperties class): all five correlations sourced, consistent with Benes & Konings (2009). Safe to use directly in any downstream thermal-hydraulic or material analysis.

- Critical dimension search (find_critical_radius): correctly finds R=23.6 cm, H=47.2 cm for k_eff = 1.03. The absolute dimensions are physically reasonable. Use this to generate geometry parameters for design sweeps.

- Normalised flux shape: phi1/phi1_max and phi2/phi2_max are correct spatial distributions. Any analysis that depends on the shape (peaking factor calculation, control rod worth estimation by flux weighting, detector positioning) can use these directly.

- PINN flux shape surrogate: the normalised phi1 and phi2 fields predicted by the PINN (MAE < 2%) are accurate enough to use for any shape-dependent calculation. The absolute magnitude requires applying the correct norm_factor from fix 1.

- Solver framework: the power_iteration engine, _build_diffusion_operator, and evaluate_design wrapper are structurally correct and can form the basis of the v2 field dataset generation once Defects 1-6 are applied.

- LHS sampling: generate_training_data() produces a valid stratified design over the 6D parameter space. Only the n_samples count and near-critical weighting need adjustment.

# SECTION 6 — PHYSICS FIDELITY CEILING OF THIS APPROACH

Even after all 12 defects are fixed, this solver remains a diffusion-theory code with approximate homogenised 2-group cross sections from a simple database. The fundamental fidelity ceiling is:

- Cross-section accuracy: the ENDF/B-VIII.0 microscopic XS are approximated as 2-group averaged constants without a transport calculation. The group boundaries, flux spectrum weighting, and self-shielding corrections are all implicit assumptions. The correct path is a lattice physics calculation (SCALE/TRITON or Serpent Monte Carlo) to produce homogenised 2-group constants for the actual FLiBe/UF4 composition at each temperature.

- Heterogeneity: the MSRE and MJM designs have graphite moderator blocks, fuel salt channels, and control rod thimbles. The current solver treats the core as a single homogeneous material. Any spatial heterogeneity (flux traps, control rod shadowing, fuel zoning) is invisible to this model.

- Thermal-hydraulic coupling: the single-channel T-H model with fixed delta_T=50K cannot resolve the coupled flux-temperature feedback that determines the actual alpha_T value. The iterative T(r,z) outer loop in Defect 7 is necessary but not sufficient — a proper coupled calculation requires resolving salt channel flow individually.

- Neutron spectrum: the 2-group approximation collapses the full neutron energy spectrum into fast and thermal bins. For a fluoride salt reactor with significant epithermal fission (the MSRE operated in an intermediate spectrum), 4-6 groups would be more appropriate.

For a ChE 612 course deliverable and PINN training data source, the current approach is appropriate and defensible once the normalization and formula bugs are fixed. For a publication or regulatory submission, SCALE/Serpent validation is required.